Note:

This template is intended for educational purposes only, without any express or implied warranty of any kind, including warranties of accuracy, completeness, or fitness for any particular purpose. You agree that your use of the template is without any recourse whatsoever to PG&E, SCE, SDG&E, or their affiliates. The template is a draft, and anyone using this document should seek the advice of an attorney to develop appropriate ordinance language to meet its jurisdiction’s specific needs, as state and local laws may differ.

Contact the Codes and Standards Reach Codes Team at info@LocalEnergyCodes.com for additional information.

This program is funded by California utility customers and administered by Pacific Gas and Electric Company, San Diego Gas & Electric Company (SDG&E®), and Southern California Edison Company under the auspices of the California Public Utilities Commission and in support of the California Energy Commission.

Model Ordinance

# Energy Reach Code for New Single Family
 and Multifamily Buildings

Version 1.0

February 2026

## Context

This document is intended to be used in conjunction with companion materials which are available at LocalEnergyCodes.com and BayAreaReachCodes.com. These materials provide important information that is necessary to develop, adopt and implement the reach code.

## Scope

- Requires higher energy performance for new single family and multifamily buildings.

- Uses Efficiency LSC Energy as the performance metric for single family homes and Source Energy for multifamily buildings.

- Does not prohibit or prevent gas technologies.

- Extends electric readiness requirements to require plans to include a designated location for the space heat pump outdoor unit in single family buildings;

- Includes exceptions for smaller homes.

- Provides option to encourage use of Certified Energy Analysts to document compliance.

## Instructions

Jurisdictions may wish to modify elements of the ordinance, such as those marked in [bracketed blue text] and those marked with comments. When modifying the language, ensure all references are maintained and that the ordinance still meets the CEC requirements.

Amendments to the State Code appear in strikeouts (deletions) and underlines (additions). Text [in blue brackets] is for staff reference and should be removed from the final ordinance.

The draft ordinance text is provided as an example only and intended as a starting point for customization for creating an ordinance that aligns with state and local requirements.  Ensure all ordinance materials are reviewed and verified by relevant jurisdiction staff and the city/county attorney.

## Changes from Prior Versions

Please check LocalEnergyCodes.com to obtain the latest version of this document and supporting materials.

| Since Version | Change |
| --- | --- |

# Model Ordinance

ORDINANCE AMENDING THE [City/Town/County of Jurisdiction] BUILDING CODE TO REQUIRE HIGHER ENERGY PERFORMANCE FOR NEWLY CONSTRUCTED RESIDENTIAL BUILDINGS

[DELETE ALL BLUE TEXT AND FOOTNOTES]

The California Building Code, Title 24, Part 6, adopted by the [City/Town/County of jurisdiction] codified under Chapter [municipal/county code reference (if not adopted in entirety, include local code references for each section)], is amended as specified below.

# Definitions

Section 100.1(b) is modified by adding the following definitions:

[Optional] CERTIFIED ENERGY ANALYST is a person registered as a Certified Energy Analyst with the California Association of Building Energy Consultants as of the date of submission of a Certificate of Compliance as required under Section 10.103.

# Performance Requirements

## Single Family

Section 150.1(b), is modified to read as follows:

(b)	Performance approach. A building complies with the Total LSC energy and source energy performance approach if the energy consumption calculated for the proposed design building is no greater than the Total LSC energy and source energy budgets calculated for the standard design building. A newly constructed single family building complies with the Efficiency LSC energy performance approach if it has an Efficiency LSC energy compliance margin of at least [ZZ% – populate with value from Attachment 1]. An addition or alteration to a single family building complies with the Efficiency LSC energy performance approach if the energy consumption calculated for the proposed design building is no greater than the Efficiency LSC energy budget calculated for the standard design building. Energy budgets and compliance margins shall be calculated using Commission-certified compliance software as specified by Sections 10-109(c) and 10-116.

EXCEPTION 1 to Section 150.1(b)1A. For a building with dwelling units of a conditioned floor area of 1,500 square feet or less and for each of the three energy metrics, the energy consumption calculated for the proposed design building shall be no greater than the energy budget calculated for the standard design building.

[Optional] EXCEPTION 2 to Section 150.1(b)1A. If the Certificate of Compliance is prepared and signed by a Certified Energy Analyst, the Efficiency LSC compliance margin may be reduced by one percentage point but shall not be less than zero.

## Multifamily

Section 170.1 is modified to read as follows:

SECTION 170.1 – PERFORMANCE APPROACH: ENERGY BUDGETS

A building complies with the LSC energy performance approach if the LSC energy consumption calculated for the proposed design building is no greater than the LSC energy budget calculated for the standard design building using Commission-certified compliance software as specified by Sections 10-109, 10-116 and the Alternative Calculation Method Reference Manual. A newly constructed building with central water-heating systems serving multiple dwelling units complies with the source energy performance approach if it has a Source Energy compliance margin of at least [ZZ% – populate with value from Attachment 1], as calculated for the standard design building using Commission-certified compliance software as specified by Sections 10-109, 10-116 and the Alternative Calculation Method Reference Manual.

[Optional] EXCEPTION 1 to Section 170.1. If the Certificate of Compliance is prepared and signed by a Certified Energy Analyst, the compliance margin may be reduced by one percentage point but shall not be less than zero.

Section 170.2.(d)B.2.ii is modified to read as follows:

ii. 	A solar water-heating system meeting the installation criteria specified in Reference Residential Appendix RA4 and with a minimum solar savings fraction of either a. or b. below:

a. 	A minimum solar savings fraction of 0.200.35 in Climate Zones 1 through 9 or a minimum solar savings fraction of 0.350.50 in Climate Zones 10 through 16; or.

b. 	A minimum solar savings fraction of 0.15 0.30 in Climate Zones 1 through 9 or a minimum solar savings fraction of 0.30 0.45 in Climate Zones 10 through 16. In addition, a drain water heat recovery system that is field verified as specified in the Reference Appendix RA3.6.9.

# Electric Readiness

## Single Family

Section 150.0.(t) is modified to read as follows:

(t) Heat Pump Space Heater Ready.

Systems using gas or propane furnace to serve individual dwelling units shall include the following:

1. A dedicated 240 volt branch circuit wiring shall be installed within 3 feet from the furnace and accessible to the furnace with no obstructions. The branch circuit conductors shall be rated at 30 amps minimum. The blank cover shall be identified as “240V ready.” All electrical components shall be installed in accordance with the California Electrical Code.

2. The main electrical service panel shall have a reserved space to allow for the installation of a double pole circuit breaker for a future heat pump space heater installation. The reserved space shall be permanently marked as “For Future 240V use.”

3. A designated exterior location for a future heat pump condensing unit with either a drain or natural drainage for condensate.

Other Sample Ordinance Sections

Severability. If any word, phrase, sentence part, section, subsection or other portion of this amendment or any application thereof to any person or circumstance is declared void, unconstitutional, or invalid for any reason, then such word, phrase, sentence, part, section, subsection, or other portion, or the prescribed application thereof, shall be severable, and the remaining provisions of this amendment, and all applications thereof, not having been declared void, unconstitutional or invalid, shall remain in full force and effect.

CEQA. This ordinance is categorically exempt from CEQA because it is an action taken by a regulatory agency for the purpose of protecting the environment (CEQA Guidelines Section 15308). In addition, this ordinance is exempt from CEQA under the general rule, 15061(b)(3), on the grounds that these standards are more stringent than the State energy standards, there are no reasonably foreseeable adverse impacts, and there is no possibility that the activity in question may have a significant effect on the environment. The following findings are made in support of these determinations:

1.	The purpose of the implementation of a Reach Code is to reduce the amount of greenhouse gas emissions in the [Jurisdiction] that are produced from buildings.

2.	The Reach Code approval process requires that the [Jurisdiction] determines that the local standards will require buildings to be designed to consume less energy than current statewide requirements. Furthermore, the California Energy Commission approval process requires that the [Jurisdiction] make the findings as part of its approval process. Therefore, the Reach Code standards can only go into effect if they protect the environment by making buildings more efficient.

Violations. Violation of the requirements of this Chapter shall be considered an infraction of the [jurisdiction Municipal/County Code], punishable by all the sanctions prescribed in [cite local reference to infractions].

Effective Date. This ordinance shall become effective and enforceable as of [Date], upon approval of the California Energy Commission or upon the date the California Building Standards Commission (CBSC) accepts the ordinance for filing, whichever is later.

Attachment 1: Compliance Margins

The table below provides reasonable compliance margins that, consistent with state and federal regulations, can be achieved cost-effectively and allow for gas appliances. These margins will encourage, but not require, all-electric construction.

## Energy Compliance Margins by Occupancy

| Climate Zone | Utility | Single Family Homes LSC Efficiency Margin | Multifamily Source Energy Margin |
| --- | --- | --- | --- |
| 1 | PGE | 6% | TBD% |
| 2 | PGE | 8% | TBD% |
| 3 | PGE | 10% | TBD% |
| 4 | CPAU | 7% | TBD% |
| 4 | PGE | 7% | TBD% |
| 5 | PGE | 10% | TBD% |
| 5 | PGE/SCG | 10% | TBD% |
| 6 | SCE/SCG | 3% | TBD% |
| 7 | SDGE | 4% | TBD% |
| 8 | SCE/SCG | 3% | TBD% |
| 9 | SCE/SCG | 2% | TBD% |
| 10 | SCE/SCG | 2% | TBD% |
| 10 | SDGE | 2% | TBD% |
| 11 | PGE | 6% | TBD% |
| 12 | PGE | 6% | TBD% |
| 12 | SMUD/PGE | 6% | TBD% |
| 13 | PGE | 5% | TBD% |
| 14 | SCE/SCG | 8% | TBD% |
| 14 | SDGE | 8% | TBD% |
| 15 | SCE/SCG | 0.8% | TBD% |
| 16 | PGE | 15% | TBD% |

https://cabec.org/cea/